TPF-5 (443) Density Profiling System “Virtual” User Group Peer Exchange

Meeting Summary Report

November 10, 2021

PURPOSE

This series of meeting summary reports are intended to give density profiling system (DPS) stakeholders easy access to pertinent information of a 2.5-day event, including a TPF-5 (443) Project Update, Peer Exchange, and Vendor Update.  The content is linked throughout to allow for access to more detailed information in sections where the reader is especially interested in the subject matter.  The focus of the day 2 event summarized in this document was a peer-exchange designed to give various agencies, contractors, and consultants perspective on DPS use.  This was highlighted by some of the more active State members as well as verbal updates from the remaining State members.  Contractors also shared their perspectives on the technology.  The event was closed out with discussion of some of the specific challenges and potential solutions toward deployment of DPS technology.

Attendance was taken from the MS Teams Meeting participants list (see Appendix B.1 and Appendix B.2). The webinar attendance was generally 90 individuals at any given time over the course of two days.

DAY TWO - November 10, 2021 Team’s Meeting 11:00 to 4:00 PM EST

RECAP (11:00am – 11:10am)

Kyle Hoegh recapped the first day event hitting on some of the highlights of each session of the day 1 event, noting that 89 attendees participated for the whole session based on the data that was downloaded of attendance.

PRESENTATIONS

Please see presentation slides located here: DPS Pooled Fund Website

2021 State Data Collection (25 minutes each)

Presentation 1 (11:10am – 11:35am): New York DOT Data Collection, Brendan Rock and Thomas Kane (NYDOT)

Brendan Rock’s presentation slides can be found here: New York Data Collection

Brendan Rock’s presentation recording can be watched Here

- 2020

  - DPS was included on 3 jobs

  - 100 ft segments

- 2021

  - DPS included on 11 jobs

- 2022

  - Look to increase the use and include special spec on D contacts

  - The new special spec calls for 1 mile or 50% of paved area

  - Added summary sheet indicating core density, nuclear gauge and DPS measurement at Core locations

  - Reducing required number of pucks

- Developing data expectations from a QA point of view

  - Further develop the summary reports and scale back the amount of data collected to reflect acceptance style testing rather than QC testing

- Active promotion

  - NYSDOT regions

  - Contractors

  - Consultants

  - Producers

- Evaluating Trends

  - Databasing all our data to evaluate trends in the comparisons between the DPS, cores, and nuclear density measurements

- Developments

  - New resource investments in NYSDOT’s program

    - Reorganized some of the Material Bureau to allow for a work group to dedicate more substantial time to DPS and other GPR activities

  - Statistical modeling of data

    - We are in the early stages of developing our statistical model for the data

    - 100ft segment density histograms

- Challenges

  - Limited resources for data collection

    - With the increase in number of jobs this year we are starting to reach the limit of our consultant’s capacity

    - 1 consultant with 2 meters

  - Data turnaround time

    - Developing more specific summary pages puts more processing time on the consultant which will delay receiving the data

  - Developing specification requirements

  - Investigation of unexpected field data

- Brendan Rock

  - Brendan.Rock@dot.ny.gov

  - (518)-457-7834

- Tom Kane

  - Thomas.Kane@dot.ny.gov

  - (518)-457-4287

- The variation in the cores is largely to do to moisture

Presentation 2 (11:35am – 12:00pm):  Ohio DOT Data Collection, Craig Landefeld (Ohio DOT)

Craig Landefeld’s presentation can be found here: Ohio Data Collection

Craig Landefeld’s presentation recording can be watched Here

- 2021

  - COVID Issues

  - Staffing issues

  - Contractor issues

- 2 Projects

  - Only got 2 days of testing on one project

  - Didn’t make the specimens for us

- Hopefully 2022 will be an improvement

- 2021 collection procedures

  - Air/Metal calibration daily

  - AM/PM line test

  - Swerve collection pattern

  - Acceptance core collection

    - Didn’t go and lay out cores at high, medium and low

    - Used cores for acceptance

  - Lab gyratory samples

  - Long joint collection

    - Single

    - Straddle

      - Sensor 6 inches on either side of the joint

- COS-US36 project details

  - 7mi. Rural 4-lane divided

  - 447 joint density spec

  - Contractor Gyratory?

    - They were supposed to but didn’t

  - Repeat line test

    - Should be consistent

    - Days of inconsistent data with no explanation

  - Machine reading that height was off – can’t adjust

  - Core Calibration

    - Started 1 in time, 1 in distance

    - Ended up doing both with both machines

    - No consistency in the data

  - Longitudinal joint scans

    - Reasonable comparison

- FAI-664 Project details

  - 5mi. Rural 2-lane

  - 1in

  - 1.25in surface course

  - Control section

    - Type 1 surface, PG 70-22M, 20% RAP

  - High RAP

    - Balanced mix design

    - Type 1 surface, PG 70-22 w. recycling agent, 55-60% RAP

  - Repeat line test

    - Concerning data for certain days

    - Both machines for a couple days, most only used 1 machine

  - Core calibrations control section

    - Went better than previous project

    - 0.7 R2 value

  - Core calibrations high RAP section

    - 0.8 R2 value

- Machine comparison

  - MAD-70

    - M1: Mean = 6.26, Std Dev = 0.298

    - M2: Mean = 6.11, Std Dev = 0.299

- Things to consider

  - Causes of sensor variability

  - Causes of poor core calibrations

  - How do we define “comparison”?

  - Is the effect of joint enrichment an issue?

  - Could we use this as a tool to assess Void Reducing Asphalt Membrane (VRAM)?

- What’s next

  - Machine comparisons

    - Precision and Bias

    - QC/QA procedure and comparison tolerances

  - Gyratory procedures

    - Work with producers

  - Collection procedures (swerve, long. Joints)

    - Swerve/Linear

    - Longitudinal joints

  - VETA analysis

    - 2022 projects

  - Specs for Pilot/Shadow projects

Presentation 3 (12:00pm – 12:30pm): North Dakota DOT Data Collection, Jon Stork (ND DOT)

Jon Stork’s presentation can be found here: ND DOT Data Collection

Jon Stork’s presentation recording can be watched Here

- Need for continuous non-destructive testing on HMA roadways

  - GSSI PaveScan RDM 2.0

- TPF-5(443): continuous compaction assessment using DPS

  - NDDOT committed from 2020-2022

- Collection during partial closure

  - Pilot car behind final roller

- 6 projects over the state

  - RAP FAA45, 58H-28

  - RAP FAA43, 58S-28

  - RAP FAA43, 58S-28

  - RAP FAA43, 58S-28, 58H-34

  - RAP FAA43, 58S-28

- MATC trailer visited

- Dielectric to Density – ND 32

  - Low, Medium, and High voids for pucks

- Mix production – Sensitivity

  - Moring and afternoon changes with mix changes

    - 1.5% change in voids

- US 83 – Daily Comparison

  - Found good accuracy with 1-2% change

- NDDOT research findings – 2021

  - Attempting Accurate density prediction

  - The variability increased when testing thinner mixes due to underlying layers

  - Good comparison

- Working with VETA

- Tips, Tricks and Lessons Learned

  - Spare set of RDM batteries

    - 5-6 hours of collection / set

  - GPS upgrade

    - Stock unit: Garmin GLO 2

    - 95% <3m accuracy

- Fog Seal

  - When to collect?

  - MnDOT doesn’t use it

  - GSSI never encountered it

  - We use it frequently

    - Pre and post fog testing for comparison

  - Dielectric increases significantly

  - Causes delays to test pre-fog for the operation

- Joints

  - Geometry

  - Double-tacked joints

  - Haven’t had good correlation when running directly over the joint

  - Notched-wedge joint

    - Haven’t had the best correlation

- Final thoughts

  - Uniformity is key

  - Consistent HMA production/construction

  - Try it!

    - Collection itself is relatively easy

    - Lab side brings difficulties, lots of resources

- Contact info

  - Jon Stork

  - jstork@nd.gov

  - (701)-328-6903

Open Discussion (12:30pm – 1:00pm): State Updates

State update/open discussion recording can be watched Here

- Idaho DOT – Craig

  - Waiting on a report from consultant

  - Putting together a training program within the state

    - Consultant staff and DOT staff

- Maine DOT – Ulrich Amoussou-Gueno: presentation can be found Here

  - Dielectrics collected over wheel path (3tf and 9ft offset form center) are higher than dielectrics collected over the center of the lane (5ft and 7ft offset) which are higher than data collected near the edges (1ft and 11ft offset).

  - Comparison between core density and density using puck calibration equation

  - Lower dielectric and slightly wider range for the 12.5 mm rubber mix project

    - Which suggest a more uniform compaction pattern for the second project

    - 2 knock down rollers used for the first project vs 1 knock down for second project

  - What’s next?

    - Confirm the interference of radio signal on the rubber mix pucks calibration due to the proximity of a radio room to the room where the calibration was done

    - Confirm that the 115 mm thickness is the way to go

    - Establish the correlation between the core’s calibration and the pucks calibration

- Maryland DOT – Larry Riggleman

  - Fully staffed

  - 1 day on 10 separate projects

    - Confident with data collection

    - Playing around with lab technology

  - Own 1 machine

- Mississippi DOT – no comment

- Missouri DOT – Jonathan Varner

  - Loan from FHWA

  - Learning the equipment

  - Taking measurements at night is difficult

  - Plastic markers affect measurements

  - Calibration Line and swerve tests

- Pennsylvania DOT – Neal Fannin

  - Staffing issues: Like what the TPF is doing, but currently don’t have a group to do field evaluation and data processing.

- Utah DOT – Ken Talbot

  - Getting ready for next year – got equipment to use

- Washington DOT – no comment

  - Kyle Thanked Kim for feedback on workplan!

- Wisconsin DOT – Michael Bohn

  - New to Pooled Fund

  - Got loaner equipment form FHWA

  - 3 different jobs

  - Lot to ask of the labs to generate pucks

    - No correlation from pucks

  - Transporting the equipment was a challenge

- Alaska DOT – Rich Giessel (Technically contributing agency)

  - 6-7% oil, 3% AV

  - Written a spec for this type of mix 4-12 mix

  - Excited to try it next year

- Florida DOT – Guangming

  - Have the device

  - Haven’t implemented in production

- South Carolina – no comment

- Arizona DOT – Nye McCarty

  - Steep learning curve for UI

  - Attempted mix calibration from both pucks and field cores

  - Challenges

    - Using Marshall mixes

    - Additional work to get the weight and height worked out in the lab

    - More deviation that would be ideal

    - Developing curve with production pucks

  - The linear seems to work better than the exponential

    - Air voids climb quickly toward the higher end

  - Anticipated an hour process – reality was a full day of collection

    - One project was full closure

    - One project was scanned before the core drill

- Massachusetts DOT – Maggie McDonald

  - Great experience with the DPS - Great tool!

  - User friendly after setup complete (setup takes a bit of time)

  - Got calibration pucks made and used PQI reading along with distance and time

    - As much data as possible

Contractor/Consultant perspective (20 minutes each)

Presentation 4 (2:00pm – 2:20pm): Mathy Construction Data Collection, Matt Oman

Matt Oman’s presentation can be found here: Mathy Data Collection

Matt Oman’s presentation recording can be found Here

- TH21

  - Paving May 21 – June 9, 2021

  - Mill 1.5 inches

  - Place two 1.5 inch lifts

  - Daily Lab measurements

    - Used MnDOT’s equipment

    - All production pucks ~95% gmm

  - Field

    - Closed lane w/ flaggers

    - Pay item based on 500 ft segment

      - Started with joint / swerve / swerve

    - Switched process to

      - Joint / mat / mat

      - + swerves at beginning and ending of day

    - Integrated QC level evaluation of DPS, non-nuke densities, PMTP, IC, and e-Ticketing

  - Data Management

    - File naming and documentation

      - Fine-tune data naming for MnDOT CO VETA import and analysis

    - Documentation

      - Precipitation

      - Construction notes

      - Sensor temporarily removed

      - GPS errors

  - DPS data

    - Sensors randomly disconnecting

    - Hanging air calibration

      - Turned out to be something with survey wheel connection and resolved by switching sensor connected to survey wheel

    - Could not view real-time data during swerve passes

  - GPS data connectivity

    - Ended up using MnDOT’s GPS antenna and controller

  - Safety concerns on busy two-lane flagger-controlled roadway

  - Training

    - 2 trained

  - Patience

    - Data re-collection (early on swerve data verification)

    - 100% functioning

    - Weather

    - Traffic

  - Collaboration and communication

    - Lab staff

    - Paving crew

    - Field QC team

    - PM and Leadership

    - MnDOT D6 and CO

- Thoughts

  - Following cold roller = evaluation and documentation only

  - Requires highly accurate, precise and verified data components

  - Careful data management and analysis (VETA)

  - Cart configuration works for 100% coverage data collection

    - Core-specific core measurements adds substantial time and logistical issues with loading / unloading the cart

  - Nominal user training

  - Cart configuration is difficult to evaluate process control

    - Have tried with sensors over hot mat but sensors overheat

  - Can measure near cores with satisfactory correlations

  - DPS collection between intermediate and cold roller

    - Collect in time mode without saving file

    - Roller water affects measurements

- Laboratory

  - Provides very good correlations to core densities

  - Testing even just a couple pucks at different air void levels

  - Additional cost for two RDM controllers

  - Additional cost for 4th sensor for complete setup

  - Equipment needs to be at the lab prior to submerging

- Parting thoughts

  - Great documentation tool

  - Provides data analysis opportunities

  - The cart configuration is mobile but not very portable

  - Yesterday NCAT indicated that lifts <2 inches can reflect underlying material

  - Will agencies capture and quantify pavement performance benefits form DPS and incorporate into LCCA calculations?

Presentation 5 (2:20pm – 2:40pm): Wuori Consulting Inc., Bryce Wuori

Bryce Wuori’s presentation can be found here: Wuori Consulting Data Collection

Bryce Wuori’s presentation recording can be watched Here

- Continuous Asphalt Mixture Compaction Assessment using DPS

- Tasks 1: Pavement Thickness Effects on DPS

  - Asphalt Concrete (AC) slabs used

    - Originally 12 AC slabs

    - Two mixtures: 9.5 & 19 mm NMAS

    - Three thicknesses per mixture

  - 4 core locations per slab

  - 3 sensors with 3 replicates in static mode

  - 2 target Air voids:; 3% and 12%

  - 3 thicknesses: 1, 2, 3 inches

  - 3 base materials: steel – 100, concrete – 6.098, AC-1 – 4.840, AC-2 – 5.497

  - For slabs with thickness > 2 in. material beneath AC slab does not affect readings

  - Action Items

    - Re-test all cores of first slabs

    - Core and test samples from second round of slabs

    - Prepare 4 more slabs (plastic and rubber – dry process)

- Task 2

  - Certification scores

    - Development of Accuracy and repeatability scores similar to Inertial profilers (AASHTO R-56)

    - Used artificial profiles and ME modeling to evaluate the effect of profile “errors” on performance

    - Scores using Root Mean Square Percent Error…

  - Threshold agreement score

    - Accuracy: an agreement of above 92 is required. Less than 10% error

    - Repeatability: an agreement of above 94 is required. Less than 5% error

  - Action items

    - Field verification of the proposes scores

    - Development of excel application

- Task 3: Precision and Bias statement

  - Asphalt Concrete (AC) pucks

  - DPS testing was conducted on each puck

  - Field testing

    - 17 test track sections (8 for statement)

  - Assessment of agreement and concordance

    - Concordance correlation coefficient (CCC)

    - Total Deviation Index (TDI)

    - Coverage Probability (CP)

  - It is statistically proven that you can replace sensors with other sensors

  - Laboratory Statements

    - 6 Sensors and 3 Operators

    - Followed ASTM E691

    - Met h- and k- consistency statistics

- Field testing

  - 17 sections built at the NCAT TT

  - Performed

    - Lab testing and correlation

      - On most cases prepared 4 samples

      - Prepare medium-high air voids puck by removing between 100 to 400 grams

    - HDPE verification

    - Line consistency verification

    - Swerve Consistency Verification

    - Full Coverage

  - Field statements

    - 6 sensors, 8 sections

    - Joint effect

    - Followed ASTM E691

    - Met h- and k- consistency statistics

  - Action items

    - Add puck test results from MnDOT

    - Add more labs to the round robin?

    - Assessment of agreement and concordance to ALL sensors

    - Update lab correlation curves with core results

    - Analysis of full coverage and swerve results

- Project schedule

- Preliminary conclusions

Open Discussion

Presentation 6 (2:40pm – 2:45pm): Ulland Brother, Mitch Froehlich

Mitch Froehlich’s presentation recording can be watched Here

- One full closure project

- One Flagger controlled with pilot cars

- Safety concerns are a big issue

- Vendor support issue needs to be address before deployment

  - These vendors need to understand this equipment is as important as the paving if it’s used for acceptance

  - Better suited for a construction company like Caterpillar

- Attaching system to rollers would be more plausible

  - Real time data between rollers would be valuable to contractors

  - More likely contractors would by into this method rather than a pushcart

Presentation 7 (2:45pm – 2:50pm): Advanced Testing, Richard Hamilton

Richard Hamilton’s presentation recording can be watched Here

- One full closure project

- One Flagger controlled with pilot cars

- Safety concerns are a big issue

- Vendor support issue needs to be address before deployment

  - These vendors need to understand this equipment is as important as the paving if it’s used for acceptance

  - Better suited for a construction company like Caterpillar

- Attaching system to rollers would be more plausible

  - Real time data between rollers would be valuable to contractors

  - More likely contractors would by into this method rather than a pushcart

Presentation 8 (2:50pm – 3:00pm): Implementation Planning Workshop Summary and Recommendations, Steve Cooper (FHWA)

Steve Cooper’s presentation can be found here: Workshop Summary

Steve Cooper’s presentation recording can be watched Here

- Workshop summary

  - To gather stakeholder feedback

  - 2-day virtual workshop conducted on April 20-21, 2021

  - Participants in attendance represented the State highway agencies, paving industry, consultants, academia, and FHWA

  - Discussions included FHWA’s requirements for QA, progress on DPS pooled fund, and 3 lead states’ examples of efforts toward advancements of DPS

  - Participants considered potential barriers and opportunities facing implementation and discussed some strategies

  - Gaps needing to be filled

    - How to deal with correlations between dielectric and in-place density

    - Related to the previous is the precision and bias between DPS units

    - Calibrating and ensuring the units are functioning properly

    - Training was seen as significant need by a number of participants

  - System improvement

    - Simplify the software

    - Add rea-time analysis tools, current PWL

    - Need troubleshooting tools

    - Flexibility in mounting options

    - Any enhancement to make the system more repeatable

    - Need for real-time vendor support during construction

  - Research gaps

    - The cause of bias between core-based and SGC puck-based calibrations

    - Impacts of moisture

    - At what point do changes in gradation, asphalt content, RAP content, or aggregate type affect calibration?

    - Are readings affected by coarser mixes?

  - Recommendations

    - FHWA requirements for quality assurance generated good discussion

      - Certification of DPS equipment

      - Certification of operators

Presentation 9 (3:00pm – 3:30pm): Use of DPS for Acceptance – Challenges and Potential Solutions, Kyle Hoegh (MnDOT) and Dennis Dvorak (FHWA)

Kyle Hoegh and Dennis Dvorak’s presentation can be found here: DPS for Acceptance

Kyle Hoegh and Dennis Dvorak’s presentation recording can be watched Here

- Acceptance methods

  - Agency DPS

  - Contractor DPS verified by agency DPS

  - Contractor DPS verified by other method such as cores

- Minimum requirements for DPS HMA density Acceptance

  - Ensure correct equipment is used

  - Ensure sensors are consistent throughout day and project

  - Ensure dielectric to density conversion is accurate for the full range of field density

  - Ensure routine collection used for acceptance evaluation is collected correctly

  - These challenges need to be overcome and procedures developed so that agencies can accept HMA density nondestructively with the improved coverage DPS offers

- Core elements of a quality assurance program

  - Agency acceptance

  - Dispute resolution

  - Independent assurance

  - Contractor QC

  - Lab qualification

  - Personnel qualification

  - All of these are core elements to the QA program

- Challenge: ensure equipment is precise/unbiased

  - The method requires an instrument capable of

    - Scanning at a rate that allows for continuous collection behind the paving operation

    - Measuring an accurate dielectric

- Challenge: ensure field collection is consistent

  - Solution: consistency checks to make sure sensors are working properly

    - Method: make sure all sensors give the same answer when testing the same population

      - Swerve

      - Line test

      - HDPE

      - Possible future: Garolite or other material closer to AC in dielectric range

  - Questions

    - What do we do when a sensor is out of range?

      - Require recalibration

      - Can we justify standardizing the sensors based on these tests with a scalar shift

      - Revert to coring if a certain amount of consistency

    - How often does the consistency need to be evaluated?

      - Beginning and end of day enough

      - Currently specifying at leas every 4000 ft or 4 hours

    - If using contractor collected data for acceptance how often does agency random sampling need to occur?

- Challenge: ensure dielectric to density correlation is accurate

  - Questions

    - How many cores are required to develop an accurate conversion?

    - How do you specify core locations to ensure you get the full range of voids?

    - How often do you need to perform a new conversion curve?

    - How do you accept or reject your core develop dielectric to?

    - How do you create your conversion curve and test the accuracy without taking too many cores?

  - Advantages over traditional way:

    - Non-destructive

      - Proven to detect changes in mix without cutting any cores in pavement

    - Can ensure full range of air voids to develop conversion from dielectric to density

    - More stable dielectric measurement than surface reflection field measurement: higher precision and covers entire thickness

    - Easily integrated into the laboratory process

      - Minimizes field operations disruption

      - No additional staff

  - Questions: how often does the sensor consistency need to be checked?

    - Research to date shows very little variation from sensor to sensor

    - Currently specifying top, rotate 90, bottom

  - Questions: what range of densities should be tested

    - Currently using production mix design voids pucks then removing mix and gyrating the same height to hit full range of voids

  - How often should we test?

  - How do we handle high sensitivity to puck thickness?

    - Currently use gyratory readout and save 1 puck per day for thickness verification using touch probe thickness setup

      - Reasonable to have a touch probe verification and how often is it needed

      - 1mm = 1% air voids

      - Gyratory calibrated

  - How can we evaluate the accuracy of the dialectic to density conversion?

    - Ideal situation: use location already randomly selected for density acceptance and collect a distance based dielectric measurement in core mode

    - Agency must mark location and contractor can’t know until done rolling

    - Changes in paving from planned makes pre-paving random selection difficult

    - Takes time to measure with collecting

    - Alternative method: don’t worry about using existing acceptance cores and just take extra randomly throughout the day

      - Not attractive to districts

  - How do we quantitatively assess agreement between cores and predictions?

    - Currently based on 0.03 Gmb criterial

    - UNH to look at stats acceptance criteria

  - What do we do if there is observed bias between predictions and cores?

    - Revert to cores?

    - Standardize prediction to match core shift?

    - UNH to look at process to make recommendations

  - How many cores do we need to check accuracy of predictions?

- Ensure routine collection used for acceptance evaluation is collected correctly

  - Questions

    - Is GPS good enough even if high accuracy?

    - How often do we need agency sampling of contractor collected data?

    - How to handle different manufacturers?

Open Discussion – Identify Potential TWG Topics Volunteers for Quarterly 2022 Meetings

Watch the Open Discussion Here

- Single TWG group with working subcommittees.

  - openness-diversity

- No need to restrict. The more the better

- Might be helpful to display this list of areas that work is still need to try to bin those areas together.

- Hawaii: not too far along

  - Trouble with implementation

  - Inspectors are concerned about manpower and who will run it

  - Sensors were outputting lower values than expected

  - Working on resolving

  - Have 4 systems

- Ken Swedeen – ruggedness of equipment is important!!

  - The durability computes to reliability

  - Traffic safety also important

  - Elephant in the room: paying incentives to encourage the process should be tied to the improvement of this equipment?

  - Need reasonable density incentives and disincentives

- Brandon Brever (MAPA)

  - For smaller contactors who only bid 1 or 2 projects a year

    - Not enough incentive to buy the equipment

    - Heavy cost burden to hire more personnel as well

- Nye McCarty

  - Puck calibration kits

    - Is there work being done to reduce the redundancy of the process?

    - Great question for the vendors

    - Can we achieve the same accuracy if those redundancies are removed?

Questions and Comments (including chat comments)

- When you say joint cores, is that 6 inches off the joint?

  - Yes 6 inches off

- Where are your joint cores located? Directly over the core?

  - 6 inches off the joint

- Are you drilling the joint cores centered on the physical joint? Do you have vertical joints or notch wedge?

  - Notch wedge joint

  - Rich Giessel sees more variability when testing on the physical joint

  - Under the surface material affecting the results as well

- Did you also compare core results with lab calibration results?

  - No, they did not – would be interesting to try

- Brendan, I would recommend using 150-ft data sublots, if possible, to match up with the Paver Mounted Thermal Profile (PMTP) method sublot sizes. The statistics per sublot can be calculated quickly in Veta per data sublot and the entire data lot. Additionally, you can visually map the data in Veta.

  - Here's an example map of DPS, IC and PMTP data mapped in Veta.

  - Used VETA for IC work not for this yet

- Slide 6: how did you run the repeat line test?

- Slide 7: would you be able to separate out which sensors were used on the core calibration measurements to see how they performed for consistency on the line or swerve test?

- Height adjustments of the sensor mount bar are possible by adjusting the alan screw in between the bar segments- we've had to do this to get dielectric values within an acceptable range

- slides 6 and 11: are you able to do intersensor comparison. Was the same sensor consistently higher or lower? Maybe see if swerve tests you ran show similar trend to line tests?

- Jon: Do you remember what the change in mix was between AM and PM? Great data there!

  - 1hr delivery time from plant to paver

  - Extended paver stops that could’ve had an affect

- Do you remember what was under the 1.5" thick lift that had a 1.49 deviation?

  - Cold and placed recycling job

- We haven't had to test on Fog seal yet at MnDOT

- We tried several sealers and fog seals but the skid numbers always tank for a few weeks and the wasn't enough performance improvements to justify their use.

- ASTM D2995 is a good method to get your tack application more consistent. I recently wrote a test method for this

- Why do they require a fog seal?

  - District elected choice 6/8 do it

  - Required when there’s low density

- Idaho does not use fog seals on our new HMA pavements.

- What were you using for your thickness input for the Gyratory pucks, Ulrich?

  - We use the a caliper for the pucks thickness measurement

- Can you think any thing in the 1st room affected the dielectric measurement?

  - Dai, Yes, As I said, there is a radio room close to the room where we used to perform the lab measurement. For some logistics reasons we decided to do the lab measurement of the second project somewhere else and we got a much better calibration. So we started looking for the reasons why we were not getting good results for the first projects and came to realize that there is a radio room close and this could be the reason. But we still need to confirm.

- Fresh off the press.... MATC will be on-site in Florida from Feb 4-27, 2022

- Rich - how many gyrations for your new mix?

- Opportunity for FDOT for some side-by-side comparison testing with the DPS... interest? Let Howard Moseley know in FDOT SMO

- Thank you to all the state DOTs who presented this morning - it was really helpful

- Thanks Kyle and panel, great information

- Thank you Kyle...great meeting

  - Thanks Eyoab!

- I was muted when I tried to speak at the end of the previous session. I wanted to say thanks to Kyle for agreeing to be a speaker at the SEAUPG meeting next week. He and Rob Sommerfeldt are presenting.

  - Thanks Wayne!

- Lunch Break

- You can just run swerve the old way as a distance file

- The patience of the operator is key to working through the bugs for sure...

- did you see interference having the sensors so close when testing the unrolled mat?

- Did you use the same antenna for lab scan that you used for core scans and if not was any correction applied?

  - It doesn’t matter what sensor you use

- Matt, any notes on asphalt plant production? For example, when the plant had to burn off higher than usual aggregate moisture which might lead to some increase in asphalt binder absorption or when there was more significant bin or gradation changes. If so, did you notice an increase in variability?

  - We can look into that

- All these different types of mixes may be of interest for the UNH study. It sounds like you have some pucks saved

- We had the same issue with cold temps 2yrs ago and had to take to a garage to warm it up!

- I noticed that the error for antennas disconnecting went away when letting the unit warm up longer

- Any feel on the impact on the tolerances of gyratory molds (some may still be within spec, but are near the upper threshold and increase bias / variability)?

  - The quick answer is that more research is needed to that and many other things

  - Robert, excellent question about gyratory molds. The quick answer is that the molds used for laboratory Quality management and are correlated with the agency's must be used for calibrating the DPS. Mold changes will change the measured air voids. However, the DPS calibration is based on measured air voids.

- It is important to understand how many cores are absolutely needed for accuracy, but are still at a manageable number (from a practicality purpose)

- Thanks Dennis & Erv

- Single TWG group with working subcommittees.

- Very good meeting. I learned a lot through the discussions that I was able to hear. The GPR technology is the way of the future, in my opinion, and will be one of the cost effective means for forensic investigations, as well as QA.

- Might be helpful to display this list of areas that work is still need to try to bin those areas together.

- openness-diversity

- No need to restrict. The more the better

- I would like to listen to the recording for the sessions that I missed yesterday. Can you provide a link to the recordings?

  - Harold Von Quintus (Guest) we will have them up in a week or so. The link will be on the website and I'm sure Kyle will also send it out as a follow-up

  - http://www.dot.state.mn.us/materials/dps/index.html

- Currently we do not have any asphalt rubber AC projects paving, but I think we have one coming up. If we still have the DPS equipment at that time, I'll make it a point to use it on that project.

- Kyle B: The MATC is available to assist Hawaii and we will be talking with Lisa Powell in the FHWA-HI next week to share what we can do to support HDOT if there's interest

- Yes, sign me up, but I am nearing retirement. I think this effort is extremely important.

- Have to sign off right now for another meeting. Again great meeting.

- Absolutely Ken ruggedness and reliability needs improvement

- What is the upper speed limit for good data collection? Will this be a limiting factor when mounting to vehicles?

  - The last comment was about ruggedness. re: speed, it can go up to 30 mph but we need to look at precision

- I know that was a big comment from Mitch at Ulland Brothers Construction

- Have sign out good meeting!

- I thought the current sensor frequencies won't support high speeds

  - If you go in "vehicle speed" in settings, you can go faster but it does affect precision of measurement.

- Great comments Ken!

- I agree. Good feedback Ken

- if incentives, QA requirements even more important for federally funded projects

- Our GPR program is missing the "vehicle speed" option. Is this part of an update?

  - should be there follow up with me later

- Excellent PEER Exchange. Thanks for all the presenters and for Kyle, Dai and Lauren

- Good job Kyle

- Excellent Meeting. Thanks

- Great meeting, thank you all

TWG Email participants

- TWG email: kyle.hoegh@state.mn.us

- TWG email: craig.landefeld@dot.ohio.gov

- TWG email: ulrich.amoussou-gueno@maine.gov

- TWG email: carliek@wsdot.wa.gov

- TWG email: dennis.dvorak@dot.gov

- TWG - flyereld@gmail.com

- TWG, bbrever@mnapa.org

- TWG, matt.oman@mathy.com

- TWG: sawalker@dot.ga.gov

- TWG richard.giessel@alaska.gov

- TWG: jonathan.varner@modot.mo.gov

- TWG: robert.hinman@dot.gov

- TWG: lriggleman@mdot.maryland.gov

- TWG: lorin.willett@dot.gov

- TWG: rich@advancetesting.com

- TWG: jstork@nd.gov

- TWG: kyle.l.buyuan@hawaii.gov

- TWG: abaskin@dot.ga.gov

- TWG: NMcCarty@azdot.gov

- TWG: nagel_brian_d@cat.com

- TWG: Julia.miller@dot.Ohio.gov

- bryce.w.wuori@gmail.com

Appendix A: Peer Exchange Agenda

Day 1:  November 9th, 2021 (All times Eastern)

| Start Time (ET) | Subject | Presenter(s) |
| --- | --- | --- |
| 11:00 | Agenda Overview and Purpose of Meeting | Shongtao Dai (MnDOT) |
| 11:10 am | Welcome and Opening Comments | Glenn Engstrom (MnDOT) |
| 11: 15 am | Update on the TPF Workplan Progress | Shongtao Dai (MnDOT) |
| 11:40 am | TPF Website Update – Live Progress Report and Analysis Tools | Kyle Hoegh (MnDOT) |
| 11:55 pm | Current DPS protocols (Draft AASHTO Spec) | Kyle Hoegh (MnDOT) |
| 12:40 pm | Protocol Review, Statistical Analysis, and Technical Brief Development Workplan | Eshan Dave (UNH), presenter Jo Sias (UNH) Adam Hand (Consultant) |
| 1:00 pm | Lunch and Catch Up on Emails |  |
| 2:00 pm | MnDOT Data Collection and Analysis | Kyle Hoegh (MnDOT) |
| 2:40 pm | MATC and Turner Fairbanks Updates | Leslie Myers (FHWA) Hoda Azari (FHWA) |
| 3:00 pm | CTC Associates Introduction | Brian Hirt (CTC) |
| 3:05 pm | NCAT Workplan Progress Update | Fabricio Leiva (NCAT) |
| 3:50 pm | Day 1 Wrap-up and Discussion | Shongtao Dai (MnDOT) |
| 4:00 pm | Adjourn Day 1 |  |

Day 2:  November 10th, 2021 (All times Eastern)

| Start Time (ET) | Subject | Presenter(s) |
| --- | --- | --- |
| 11:00 am | Day 1 Recap | Kyle Hoegh (MnDOT) |
| 11:10 am | 2021 State Data Collection (25 minutes each) | New York DOT – Thomas Kane Ohio DOT – Craig Landefeld North Dakota DOT – Jon Stork |
| 12: 25 pm | Open Discussion – State Updates | Pooled Fund and MATC States |
| 1:00 pm | Lunch and Catch Up on Emails |  |
| 2:00 pm | 2021 Contractor/Consultant Perspective (20 minutes each) | Matt Oman (Mathy) Bryce Wuori (Wuori Construction) |
| 2:40 pm | Contractor/Consultant Open Discussion | Mitch Froehlich (MN), Advanced Testing (NY) |
| 2:50 pm | Implementation Planning Workshop Summary and Recommendations | Steve Cooper (FHWA) |
| 3:00 pm | Use of DPS for Acceptance (challenges and potential solutions) | Dennis Dvorak (FHWA) Kyle Hoegh (MnDOT) |
| 3:30 pm | Open Discussion (Identify potential TWG topics volunteers for quarterly 2022 meetings) | Kyle Hoegh (MnDOT) Shongtao Dai (MnDOT) Brian Hirt (CTC) |
| 4:00 pm | Adjourn Day 2 |  |