Office of Traffic Engineering

Waters Edge, 1500 W. County Rd B2

Roseville, MN  55113

Frequently Asked Questions and Answers

for Square Tube Sign Structure Installations

- Has the 3 wall base passed MASH-16 crash testing?

The 3 wall base has been determined to function similar to the single post base that was successfully crash tested.

- What are the typical uses for the 3 wall base?

The 3 wall base can be used for markers, delineators, and small chevrons.  The 3 wall base can only be used in single post installations.

- Has the Fin base passed MASH-16 crash testing?

The Fin base has been determined to function similar to what was successfully crash tested.

- Can the Fin base be used for multi-post installations?

The Fin base can only be used in single post installations.

- Has the Shear Bolt base passed MASH-16 crash testing?

The Shear Bolt base has not passed crash testing.  It is currently being tested.

- Can the Shear Bolt base be used after December 31, 2019?

The Shear Bolt base can be used after December 31, 2019.  The base was redesigned and is currently being tested.  FHWA allows the use of the best available device to meet a need. There is a need for a device to mount signs in areas covered by 4 inch concrete.  This device meets that need and can be used until a device is successfully MASH-16 tested that can meet that need.

- Has the Slip base passed MASH-16 crash testing?

The Slip base uses a thicker gauge post and an insert.  Th Slip base has been determined to function similar to the thinner gauge post without an insert that was successfully crash tested.

- Can the Slip base be used for multi-post installations?

The slip base can be used for both single post and multi-post installations.  Multi-post installations cannot be used in intersections because the multi-post was not crash tested at 90 degrees.  An intersection area is defined as everything within the intersection out to 50 feet beyond the edge of road radii.

- Has the MASSH 400 Slip base been MASH-16 crash tested?

The MASSH 400 Slip base has been successfully crash tested.  This is a good alternative for smaller I-Beam signs.

- Has the I-Beam sign been MASH-16 crash tested?

- The I-Beam sign was successfully crash tested and can be used.

- Has u-channel post been MASH-16 crash tested?

Missouri was crash testing their u-channel design, but we have not seen any results.  Missouri’s design has a splice closer to the ground than ours.

- Will the u-channel design with a structural splice above ground be tested in the future?

- The u-channel may get tested as part of the Midwest Roadside Safety Pooled Fund.

- Will we be able to use a single post u-channel for delineators after December 31, 2019?

- No, but there’s a good possibility it will get tested through the Midwest Roadside Safety Pooled Fund. The three-wall base will have to get used until the u-channel gets tested.

- What type of post base can be used in an intersection?

Any type of base except 4 inch slip base can be used in an intersection.  All bases except 4 inch slip base were tested at 0 and 90 degrees successfully for a single post.  Do not install any multi-post signs in the intersection area.

- How do we handle sign assemblies that are too large for a single post in intersections?

A typical sign assembly that includes back to back one way signs, a stop or yield sign, and a divided highway sign can be installed on a single post.  Because these signs are not a uniform width, we analyzed the assembly and determined it would meet the wind loading requirements of a single post.  Contact the Office of Traffic Engineering with any other unique sign assemblies you have in an intersection that fall outside of the 1 post area on the wind loading charts.  We will review them.

- Can a no passing zone sign be installed on a single post?

The no passing zone sign can be installed on a single slip base support.

- Are there standard plans for these types of structures?

There is a standard plan sheet for the 3 wall base design. There are detail sheets for the other square tube sign structures.

- Will there be training on how to include these new sign standards on plans?

- OTE is providing training on this in the Signing Plan Design: Conventional Roads class.

  - Where can I find a sample plan, wind loading charts and other signing design tools?

- All signing design tools can be found on the Signing website http://www.dot.state.mn.us/trafficeng/signing/plans.html

- Contact the Signing Unit for design tools not listed on this website.

  - Where can I find the punching codes?

Punching codes are included in the Standard Signs and Markings Manual on the Signing website

http://www.dot.state.mn.us/trafficeng/publ/signsmanual/index.html

  - What are my options if there is a concrete surface where the sign will be placed?

For a shear bolt base – The concrete must be at least 4 inches thick and the surface slope must be less than 8%.  The minimum mounting height is 7 feet.

For a slip base – The concrete must be at least 8 inches thick and the surface slope must be less than 8%.  The minimum mounting height is 7 feet.

Core the concrete for all signs mounted at less than 7 feet (i.e. markers and delineators) or if the surface slope for 7 foot mounted signs is greater than 8%.

  - What post should be used for an electrically powered sign?

For solar panel powered signs use 2 ½” square tube slip base.

For hard wired (buried cable) powered signs use a pedestal design similar to ones used for signals.

If your questions were not addressed, contact Steve McBurney at steven.mcburney@state.mn.us.